Name: RTL JOULES OPT TOOLS

githubUrl: https://github.com/Vishnukamali3/RTL_JOULES_OPT_TOOLS

description: This project focuses on the design and verification of an 8-bit Universal Shift Register using Verilog HDL and Vivado. The design supports multiple operations such as Hold, Shift Left, Shift Right, and Parallel Load through mode selection signals. Along with functional verification, the project performs simplified PPAC (Power, Performance, Area, and Congestion) analysis using switching activity estimation, synthesis reports, and timing analysis. The project demonstrates optimization-aware RTL design concepts inspired by industrial Joules RTL optimization methodologies in an FPGA-based environment.

### Functional Requirements

### RTL Power Analysis

- Cadence Joules analyzes power consumption at the RTL stage of digital design. It helps    designers estimate power usage early before chip fabrication.

### Switching Activity Detection

- The tool identifies unnecessary switching activity in circuits. This helps reduce dynamic power consumption and improve efficiency.

### Clock Gating Support

- Joules supports clock gating techniques to control unnecessary clock signals. This reduces power wastage in idle circuit blocks.

### Power Report Generation

- The tool generates detailed power analysis reports. These reports help designers understand power distribution in the design.

### High-Power Block Identification

- Joules detects modules that consume excessive power. Designers can optimize these blocks for better performance and lower energy usage.

### Design Optimization

- The tool helps improve power, performance, and area (PPA). It supports efficient low-power semiconductor chip design.

### Non-Functional Requirements

- Non-Functional Requirements of Cadence Joules

- Perform power analysis quickly and efficiently.

- Provide accurate and reliable power estimation results.

- Support both small and large-scale VLSI designs.

- Operate consistently without crashes or data loss.

- Provide an easy-to-use interface and clear reports.

- Integrate smoothly with RTL, synthesis, and simulation tools.

- Maintain stable performance during complex design analysis.

Support standard VLSI design flows and file formats.

problem_statement: Modern digital systems require efficient RTL designs that achieve low power consumption, high performance, reduced hardware area, and optimized switching activity. Traditional educational RTL projects mainly focus on functionality verification and often ignore PPAC (Power, Performance, Area, and Congestion) analysis used in industrial ASIC and FPGA design flows. There is a need for a simplified and FPGA-accessible methodology to study industrial RTL optimization concepts using standard tools like Vivado. Therefore, the challenge is to design an 8-bit Universal Shift Register capable of multiple data operations while also analyzing its PPAC characteristics through RTL-level estimation and synthesis reports.

proposed_solution: To address this problem, an 8-bit Universal Shift Register is designed using Verilog HDL with support for Hold, Shift Left, Shift Right, and Parallel Load operations through mode selection signals. The design is simulated and verified in Vivado to ensure correct functionality. A simplified PPAC analysis methodology inspired by Joules RTL Design Studio is implemented using toggle-rate based switching activity estimation, mode activity monitoring, timing analysis, and FPGA resource utilization reports. Optimization-aware RTL techniques such as clock-enable based activity reduction and structured coding are incorporated to reduce unnecessary switching activity and improve overall design efficiency. Vivado synthesis and implementation reports are used to evaluate power, timing, area utilization, and congestion-related behavior in an educational FPGA environment.

technologies_used: Verilog HDL, Vivado Design Suite, Xilinx FPGA Flow, RTL Design Methodology, PPAC Analysis Techniques, VCD Switching Activity Analysis, Behavioral Simulation, Synthesis and Timing Analysis, and Optimization-Aware RTL Coding.

system_architecture: The system architecture consists of an 8-bit Universal Shift Register designed using Verilog HDL, controlled by clock, reset, mode selection signals, serial inputs, and parallel data inputs. The architecture supports four operations: Hold, Shift Left, Shift Right, and Parallel Load. A control logic block selects the required operation based on mode inputs. The RTL design is simulated and synthesized in Vivado, where switching activity, timing performance, power consumption, and area utilization are analyzed using VCD files and synthesis reports. The PPAC analysis block estimates Power, Performance, Area, and Congestion characteristics to study optimization-aware RTL behavior.

Outputs:

Waveforms:

Performance:

Area and power:

Congestion:

in_scope: The scope of this project includes the design, simulation, synthesis, and PPAC analysis of an 8-bit Universal Shift Register using Verilog HDL and Vivado. The project focuses on studying power consumption, timing performance, area utilization, and switching activity using FPGA-based RTL design techniques. It also demonstrates optimization-aware coding methods inspired by industrial Joules RTL optimization flows, providing practical understanding of RTL verification and hardware efficiency analysis in an educational environment.

out_scope: The project does not include ASIC fabrication, advanced physical design implementation, real-time hardware prototyping, or detailed industrial congestion analysis. It also excludes advanced low-power techniques such as clock gating, power gating, dynamic voltage scaling, and machine learning-based optimization methods. The work is limited to FPGA-based RTL-level PPAC estimation and functional verification using Vivado tools.

future_enhancements:

Enhancement 1 — Committed Vivado Synthesis and Implementation Reports: Run the provided run_all.tcl script and commit the resulting Vivado reports (timing summary, utilization, and power) to the repository under the reports/ folder. This converts PPAC claims from expected commentary in source files into concrete, independently verifiable evidence — positive vs. negative WNS, exact LUT/FF counts, and mW dynamic power figures — making the industrial-style analysis fully reproducible.

Enhancement 2 — VCD-Based Exact Power Analysis: Extend the existing run_all.tcl script to read the sim_dump.vcd file generated by the testbench and invoke Vivado’s read_vcd and report_power commands after implementation. This replaces the toggle-count proxy approach in the testbench with exact mW dynamic power values from Vivado’s internal power model, providing a direct comparison of dynamic power between the two RTL styles.

Enhancement 3 — Formal Linting and Static Analysis: Integrate a formal linting pass using Verilator or SpyGlass into the project flow. This would catch clock-domain violations, unintentional latches, and reset-fanout issues automatically, adding a professional verification discipline layer beyond simulation and synthesis reports.

Enhancement 4 — Multi-Width PPAC Scaling Study (16-bit / 32-bit): Both RTL modules already use a parameterized WIDTH. Instantiate 16-bit and 32-bit variants and re-run the full synthesis and implementation flow. Comparing LUT count, FF count, timing slack, and dynamic power across bit-widths demonstrates how PPAC trade-offs scale with data width and provides quantitative evidence of design portability.

Enhancement 5 — CI/CD Pipeline with Automated Regression: Add a GitHub Actions workflow that runs xsim simulation on every push, validates the Passed=9 Failed=0 result against sim_output.log, and posts a build badge to the repository. This closes the gap between the documented expected output and a live, continuously verified result — making the PPAC claims fully auditable without requiring a local Vivado installation.

conclusion: This project demonstrates that RTL coding style has measurable, quantifiable impact across all four PPAC dimensions simultaneously. By contrasting an intentionally non-optimized 8-bit Universal Shift Register against an optimization-aware counterpart on the same Artix-7 device, the study shows that a synchronous reset wastes a LUT that an asynchronous reset reclaims as a dedicated CLR pin; a separate combinational always block adds a critical-path logic level that a merged clocked block eliminates; registered serial outputs consume flip-flops that wire taps avoid entirely; and the absence of a clock enable forces unnecessary FF switching that a CE pin suppresses. These are not merely theoretical FPGA micro-optimizations — they are the same RTL design principles that industrial tools such as Cadence Joules RTL Design Studio enforce at scale. The project equips practitioners with a hands-on, tool-accessible framework for understanding why optimization-aware RTL coding matters, and provides a fully reproducible Vivado flow to verify every PPAC claim independently.

projectType: FPGA-Based RTL Design and PPAC Analysis Project using Verilog HDL and Vivado.